Measured Hydraulic Conductivities at the Wells G & H Superfund Site
(compiled by Maura Methney from U.S. EPA subcontractor reports)
	
	
	
	  Hydraulic Conductivity

	Material

Type
	  Location
	Sediment Description
	Number of Tests
	Min

(ft/d)
	Max

(ft/d)
	Geometric Mean

(ft/d)

	Overcompacted
	Northeast
	Sandy silt, gravelly silt
	19
	0.01
	1.1
	0.2

	
	
	Diamict
	3
	0.02
	0.1
	0.05

	
	Southwest at BW9
	Diamict
	3
	0.6
	200
	37

	
	Site-wide
	Silt
	1
	0.05
	0.05
	0.05

	
	
	Sandy silt
	20
	0.01
	3.6
	0.2

	
	
	Silty sand
	3
	0.1
	2.6
	0.8

	
	
	Diamict
	13
	0.02
	77
	3

	
	
	Gravelly sand, sandy gravel, sand
	13
	0.1
	58
	0.7

	Loose
	Site-wide
	Silt, sandy silt, sandy clay
	3
	0.43
	12.5
	1.7

	
	
	Silty sand
	5
	0.07
	260
	0.9

	
	
	Diamict
	2
	2.0
	160
	18

	
	
	Sand
	28
	0.1
	280
	7

	
	
	Gravelly sand, sandy gravel
	14
	0.25
	350
	10

	Bedrock
	Northeast
	
	29
	0.002
	5.2
	0.1

	
	Southwest
	
	9
	0.01
	43
	4

	
	Site-wide
	
	41
	0.002
	43
	0.3


